Q&D CheckIOPorts v1.3c by PhG

Syntax: CHKIO <option>... [ > file ]

Here are available options (without '-' or '/' characters) :

- <port> [count] checks 'count' I/O ports starting from 'port'

'port' belongs to the [$0000..$03FF] range, 'count' default value is 16

Values are given in decimal, unless they begin with a '$' for hexadecimal

- FREE shows the most probably unused I/O ports in the [$0200..$03FF] range

- USED shows the most probably used I/O ports in the [$0200..$03FF] range

- MAP shows a map of I/O ports in the [$0200..$03FF] range

- SAMPLE <ticks> <port> [count] install program as a TSR

'ticks' is the sampling frequency (one tick is 1/18.2s)

Note about 400Kb EMS memory is required for TSR mode.

- RESET reinitializes results got from TSR

- REPORT dumps current results to CHKIO.RPT file

Previous file of same name is kept as CHKIO.BAK

- STATUS shows current TSR status

- UNLOAD tries and unloads program from memory

For SAFETY, FREE and USED commands do NOT test hard disk and floppy I/O ports.

Program can *try* 4 methods to check whether I/O ports are used or not.

To specify survey method, enter M0, M1, M2 or M3 as the *first* parameter.

Default is M0. Beware: once program is TSR, you CANNOT change survey method!